A (Very) Brief Tutorial for the MainStem Version of DHSVM-RBM

The source code, support code and documentation can be obtained with the git command:

git clone https://githubm.com/jyearsley/DHSVM-RBM.git

The material for setting up the Connecticut River main stem problem is on the repository of the MainStem branch: Input Data

Input Data:

Conn.Main_Stem (originally, Conn.Main_Stem3)

– Contains the mapping from main stem RBM segments to main stem DHSVM segments, ordinal index and segment length.

Conn.Trib (originally, ConnMain.Trib2)

– Contains the mappings from RBM and DHSVM main and tributary segments to ordinal indices for RBM main segments and tributaries.

Conn. Segments

– Header of indices from the main.stem.*.Only files that define the order in which data are arrayed in the main.stem.*.Only files.

Conn.forcing

– Heat budget and hydrology forcing data developed by running the original version of Create_File.

trib.Temp.Only

– File with water temperatures of the tributaries

trib.Inflow.Only

– File with stream flows of the tributaries

Conn.Only_Table

– Header of indices from the trib.*.Only files that define the order in which data are arrayed in the trib.*.Only files.

Source Code

build_main_network.pl

– Perl code to build the network file *.net and the *.Source file

Create_File.f90

– Fortran 90 source code to create the Conn.tribs file, the file with the tributary flows and stream temperatures. Modified from the original, Create_File.f90.

RBM.f & RBM.fi

– Fortran 90 source code, modified from original to accommodate the

Connecticut River main stem.

Process

1. After properly renaming the mapping files and copying the Trib.*.Only header to the

Conn.Only_Table and the main.stem.*.Only header to the Conn. Segments, run the Perl script, build_main_network.pl, with the project name, Conn.

perl build_main_network.pl <Project Name>

e.g.

Perl build_main_network.pl Conn

2. Create the Mohseni and Leopold parameter files, Conn.Leopold & Conn.Mohseni, with the script, write_params.pl.

3. Create Conn.forcing by running the original version of Create_File using the heat budget and hydrology forcing data (*.only).

4. Create Conn.tribs by running the modified version of Create_File. The Conn.tribs file contains the tributary flows and stream temperatures.

perl Create_File <Input directory> <Project Name>

e.g.

perl Create_File ./ Conn

<Input directory> is where *.Only files are located.

5. After creating the executable, RBM, run it as:

./RBM Conn

Output

Conn.Source

The file, Conn.Source has the following format:

Group 1:  <no_segments> total number of main stem segments

Group 2 has 7 columns:

Column 1: <basin name> -- the basin where the segment is located

Column 2: <ordinal index> of main stem segments

Column 3: <main segID> -- segment index as in the 4th column of Conn. Main_Stem file.

Column 4: <nsrc> number of sources or tributaries.

Column 5: <trib segID> -- segment index as in the 7th column of Conn.Trib file.

Column 6: <ordinal index> of tributary segments.

Conn.segmap

The file, Conn.segmap, has the following format:

<no_headwater> <no_segments>

Conn.tribs

The file, Conn.tribs, has the following format:

<inflow> <tempeature> for each tributary as in Conn.Source.

Conn.net

Conn.forcing

Remaining Issues to be fixed later:

1. When we have complete files for the whole Connecticut River network, change Create_File.f90, though.  Change the references for hydrology from "no_tribs+1" to "no_tribs".